--/--/2018

C116

SCHEDULE 5 TO CLAUSE 32.09 NEIGHBOURHOOD RESIDENTIAL ZONE

Shown on the planning scheme map as NRZ5.

JUBILEE PARK

--/--/2018

C116

1.0	Neighbourhood character objectives

To ensure that the spacious and modest residential character of Jubilee Park is respected and enhanced.

To ensure low scale building forms, consistent with the neighbourhood character of the area.

To ensure that all dwellings, their settings and front fences make a positive contribution to the public domain and pedestrian environment.

To maintain a sense of spaciousness around dwellings, when viewed from the street.

To ensure that front setbacks are not dominated by garages, carports or vehicular access.

--/--/2018

C116

2.0	Minimum subdivision area

None specified

--/--/2018

C116

3.0	Permit requirement for the construction or extension of one dwelling or a fence associated with a dwelling on a lot

|  | Requirement |
| --- | --- |
| Permit requirement for the construction or extension of one dwelling on a lot | None specified |
| Permit requirement to construct or extend a front fence within 3 metres of a street associated with a dwelling on a lot | None specified |

--/--/2018

C116

4.0	Requirements of Clause 54 and Clause 55

|  | Standard | Requirement |
| --- | --- | --- |
| Minimum street setback | A3 and B6 | None specified |
| Site coverage | A5 and B8 | None specified |
| Permeability | A6 and B9 | None specified |
| Landscaping | B13 | None specified |
| Side and rear setbacks | A10 and B17 | Side setbacks 1.2 metres, plus 0.3 metres for every metre of height over 3.6 metres up to 6.9 metres, plus 1 metre for every metre of height over 6.9 metres. Rear setbacks 5 metres, plus 0.3 metres for every metre of height over 3.6 metres up to 6.9 metres, plus 1 metre for every metre of height over 6.9 metres. |
| Walls on boundaries | A11 and B18 | None specified |
| Private open space | A17 | An area of 80 square metres, with one part of the private open space to consist of secluded private open space at the side or rear of the dwelling with a minimum area of 60 square metres, a minimum dimension of 5 metres and convenient access from a living room. |
|  | B28 | An area of 80 square metres, with one part of the private open space to consist of secluded private open space at the side or rear of the dwelling with a minimum area of 60 square metres, a minimum dimension of 5 metres and convenient access from a living room. |
| Front fence height | A20 and B32 | 0.9 metres |

--/--/2018

C116

5.0	Maximum building height requirement for a dwelling or residential building

None specified

--/--/2018

C116

6.0	Application requirements

None specified

--/--/2018

C116

7.0	Decision guidelines

The following decision guidelines apply to an application for a permit under Clause 32.09, in addition to those specified in Clause 32.09 and elsewhere in the scheme which must be considered, as appropriate, by the responsible authority:

The extent to which the proposed buildings or works meet the neighbourhood character objectives of this clause.

The extent to which any building to be demolished, extended or otherwise modified, contributes to the preferred neighbourhood character of the area.